# Copyright (c) 2000-2002 SuSE GmbH Nuernberg, Germany.  All rights reserved.

# Copyright (c) 2003,2004 SuSE Linux AG Nuernberg, Germany.  All rights reserved.

# Copyright (c) 2005-2007 SUSE LINUX Products GmbH Nuernberg, Germany.  All rights reserved.

#

# Author: Marc Heuse, 2002

#         Ludwig Nussel, 2004-2007

#

# /etc/sysconfig/SuSEfirewall2

#

# for use with /sbin/SuSEfirewall2 version 3.6

#

# ------------------------------------------------------------------------     #

# PLEASE NOTE THE FOLLOWING:

#

# Just by configuring these settings and using the SuSEfirewall2 you

# are not secure per se! There is *not* such a thing you install and

# hence you are saved from all (security) hazards.

#

# To ensure your security, you need also:

#

#   * Secure all services you are offering to untrusted networks

#     (internet) You can do this by using software which has been

#     designed with security in mind (like postfix, vsftpd, ssh),

#     setting these up without misconfiguration and praying, that

#     they have got really no holes. Apparmor can help in

#     most circumstances to reduce the risk.

#   * Do not run untrusted software. (philosophical question, can

#     you trust SuSE or any other software distributor?)

#   * Check the security of your server(s) regulary

#   * If you are using this server as a firewall/bastion host to the

#     internet for an internal network, try to run proxy services

#     for everything and disable routing on this machine.

#   * If you run DNS on the firewall: disable untrusted zone

#     transfers and either don't allow access to it from the

#     internet or run it split-brained.

#

# Good luck!

#

# Yours,

#	SuSE Security Team

#

# ------------------------------------------------------------------------

#

# Configuration HELP:

#

# If you have got any problems configuring this file, take a look at

# /usr/share/doc/packages/SuSEfirewall2/EXAMPLES or use YaST

#

#

# If you are an end-user who is NOT connected to two networks (read:

# you have got a single user system and are using a dialup to the

# internet) you just have to configure (all other settings are OK):

# 2) and maybe 9).

#

# If this server is a firewall, which should act like a proxy (no direct

# routing between both networks), or you are an end-user connected to the

# internet and to an internal network, you have to setup your proxys and

# reconfigure (all other settings are OK): 2), 3), 9) and maybe 7), 11), 14)

#

# If this server is a firewall, and should do routing/masquerading between

# the untrusted and the trusted network, you have to reconfigure (all other

# settings are OK): 2), 3), 5), 6), 9), and maybe 7), 10), 11), 12), 13),

# 14)

#

# If you want to run a DMZ in either of the above three standard setups, you

# just have to configure *additionally* 4), 9), 12), 13), 18)

#

# Please note that if you use service names, they have to exist in

# /etc/services. There is for example no service "dns", it's called

# "domain"; email is called "smtp" etc.

#

# ------------------------------------------------------------------------

## Path:	Network/Firewall/SuSEfirewall2

## Description:	SuSEfirewall2 configuration

## Type:	string

## Default:	any

#

# 2.)

# Which are the interfaces that point to the internet/untrusted

# networks?

#

# Enter all untrusted network devices here

#

# Format: space separated list of interface or configuration names

#

# The special keyword "any" means that packets arriving on interfaces not

# explicitly configured as int, ext or dmz will be considered external. Note:

# this setting only works for packets destined for the local machine. If you

# want forwarding or masquerading you still have to add the external interfaces

# individually. "any" can be mixed with other interface names.

#

# Examples: "ippp0 ippp1", "any dsl0"

#

# Note: alias interfaces (like eth0:1) are ignored

#

FW_DEV_EXT="any eth0"

## Type:	string

#

# 3.)

# Which are the interfaces that point to the internal network?

#

# Enter all trusted network interfaces here. If you are not

# connected to a trusted network (e.g. you have just a dialup) leave

# this empty.

#

# Format: space separated list of interface or configuration names

#

# Examples: "tr0", "eth0 eth1"

#

FW_DEV_INT="eth1"

## Type:	string

#

# 4.)

# Which are the interfaces that point to the dmz or dialup network?

#

# Enter all the network devices here which point to the dmz/dialups.

# A "dmz" is a special, seperated network, which is only connected

# to the firewall, and should be reachable from the internet to

# provide services, e.g. WWW, Mail, etc. and hence is at risk from

# attacks. See /usr/share/doc/packages/SuSEfirewall2/EXAMPLES for an

# example.

#

# Note: You have to configure FW_FORWARD to define the services

# which should be available to the internet and set FW_ROUTE to yes.

#

# Format: space separated list of interface or configuration names

#

# Examples: "tr0", "eth0 eth1"

#

FW_DEV_DMZ=""

## Type:	yesno

## Default:	no

#

# 5.)

# Should routing between the internet, dmz and internal network be

# activated?

#

# Set this to "yes" if you either want to masquerade internal

# machines or allow access to the dmz (or internal machines, but

# this is not a good idea).

#

# This option overrides IP_FORWARD from

# /etc/sysconfig/network/options

#

# Setting this option one alone doesn't do anything. Either activate

# masquerading with FW_MASQUERADE below if you want to masquerade

# your internal network to the internet, or configure FW_FORWARD to

# define what is allowed to be forwarded. You also need to define

# internal or dmz interfaces in FW_DEV_INT or FW_DEV_DMZ.

#

# defaults to "no" if not set

#

FW_ROUTE="yes"

## Type:	yesno

## Default:	no

#

# 6.)

# Do you want to masquerade internal networks to the outside?

#

# Requires: FW_DEV_INT or FW_DEV_DMZ, FW_ROUTE, FW_MASQ_DEV

#

# "Masquerading" means that all your internal machines which use

# services on the internet seem to come from your firewall. Please

# note that it is more secure to communicate via proxies to the

# internet than to use masquerading.

#

# This option is required for FW_MASQ_NETS and FW_FORWARD_MASQ.

#

# defaults to "no" if not set

#

FW_MASQUERADE="yes"

## Type:	string

## Default:     zone:ext

#

# 6a.)

# You also have to define on which interfaces to masquerade on.

# Those are usually the same as the external interfaces. Most users

# can leave the default.

#

# The special string "zone:" concatenated with the name of a zone

# means to take all interfaces in the specified zone.

#

# Old version of SuSEfirewall2 used a shell variable ($FW_DEV_EXT)

# here. That method is deprecated as it breaks auto detection of

# interfaces. Please use zone:ext instead.

#

# Examples: "ippp0", "zone:ext"

#

FW_MASQ_DEV="zone:ext"

## Type:	string

## Default:	0/0

#

# Which internal computers/networks are allowed to access the

# internet via masquerading (not via proxys on the firewall)?

#

# Format: space separated list of

#  <source network>[,<destination network>,<protocol>[,port[:port]]

#

#  If the protocol is icmp then port is interpreted as icmp type

#

# Examples: - "0/0" unrestricted access to the internet

#           - "10.0.0.0/8" allows the whole 10.0.0.0 network with

#             unrestricted access.

#           - "10.0.1.0/24,0/0,tcp,80 10.0.1.0/24,0/0,tcp,21" allows

#             the 10.0.1.0 network to use www/ftp to the internet. -

#           - "10.0.1.0/24,0/0,tcp,1024:65535 10.0.2.0/24" the

#             10.0.1.0/24 network is allowed to access unprivileged

#             ports whereas 10.0.2.0/24 is granted unrestricted

#             access.

#           - "0/0,!10.0.0.0/8" unrestricted access to the internet

#             with the exception of 10.0.0.8 which will not be

#             masqueraded.

#

FW_MASQ_NETS="0/0"

## Type:	string

## Default:	0/0

#

# Which computers/networks should be excluded from beeing masqueraded?

# Note that this only affects the POSTROUTING chain of the nat

# table. Ie the forwarding rules installed by FW_MASQ_NETS do not

# include the listed exceptions.

# *** Since you may use FW_NOMASQ_NETS together with IPsec make sure

# that the policy database is loaded even when the tunnel is not up

# yet. Otherwise packets to the listed networks will be forwarded to

# the internet unencrypted! ***

#

# Format: space separated list of

#  <source network>[,<destination network>,<protocol>[,port[:port]]

#

#  If the protocol is icmp then port is interpreted as icmp type

#

# Examples: - "0/0,10.0.0.0/8" do not masquerade packets from

#             anywhere to the 10.0.0.0/8 network

#

FW_NOMASQ_NETS=""

## Type:	yesno

## Default:	no

#

# 7.)

# Do you want to protect the firewall from the internal network?

# Requires: FW_DEV_INT

#

# If you set this to "yes", internal machines may only access

# services on the firewall you explicitly allow. If you set this to

# "no", any internal user can connect (and attack) any service on

# the firewall.

#

# defaults to "yes" if not set

#

# see also FW_REJECT_INT

#

FW_PROTECT_FROM_INT="yes"

## Type:	string

#

# 9.)

# Which TCP services _on the firewall_ should be accessible from

# untrusted networks?

#

# Enter all ports or known portnames below, seperated by a space.

# TCP services (e.g. SMTP, WWW) must be set in FW_SERVICES_*_TCP, and

# UDP services (e.g. syslog) must be set in FW_SERVICES_*_UDP.

# e.g. if a webserver on the firewall should be accessible from the internet:

# FW_SERVICES_EXT_TCP="www"

# e.g. if the firewall should receive syslog messages from the dmz:

# FW_SERVICES_DMZ_UDP="syslog"

# For IP protocols (like GRE for PPTP, or OSPF for routing) you need to set

# FW_SERVICES_*_IP with the protocol name or number (see /etc/protocols)

#

# Format: space separated list of ports, port ranges or well known

#         service names (see /etc/services)

#

# Examples: "ssh", "123 514", "3200:3299", "ftp 22 telnet 512:514"

#

FW_SERVICES_EXT_TCP="5801 5901 domain http https imap imaps ipp iscsi-target ldap ldaps microsoft-ds mysql netbios-ssn pbs pbs_mom pbs_resmom pbs_sched pop3 pop3s rsync smtp ssh svrloc xdmcp"

## Type:	string

#

# Which UDP services _on the firewall_ should be accessible from

# untrusted networks?

#

# see comments for FW_SERVICES_EXT_TCP

#

# Example: "53"

#

FW_SERVICES_EXT_UDP="110 bootpc domain ipp ipsec-nat-t isakmp netbios-dgm netbios-ns ntp pbs_resmom svrloc tftp xdmcp"

## Type:	string

#

# Which UDP services _on the firewall_ should be accessible from

# untrusted networks?

#

# Usually for VPN/Routing which END at the firewall

#

# Example: "esp"

#

FW_SERVICES_EXT_IP="Porta esp personalizada"

## Type:        string

#

# Which RPC services _on the firewall_ should be accessible from

# untrusted networks?

#

# Port numbers of RPC services are dynamically assigned by the

# portmapper. Therefore "rpcinfo -p localhost" has to be used to

# automatically determine the currently assigned port for the

# services specified here.

#

# USE WITH CAUTION!

# regular users can register rpc services and therefore may be able

# to have SuSEfirewall2 open arbitrary ports

#

# Example: "mountd nfs"

FW_SERVICES_EXT_RPC="fypxfrd mountd nfs nfs_acl nlockmgr portmap sgi_fam status ypbind yppasswdd ypserv"

## Type:        string

#

# Which services _on the firewall_ should be accessible from

# untrusted networks?

#

# Packages can drop a configuration file that specifies all required

# ports into /usr/share/SuSEfirewall2/services . That is handy for

# services that require multiple ports or protocols. Enter the space

# separated list of configuration files you want to load.

#

# Example: "samba-server nfs-server"

FW_CONFIGURATIONS_EXT="avahi cups dhcp-server sshd vnc-httpd vnc-server xorg-x11-Xvnc ypbind"

## Type:	string

#

# see comments for FW_SERVICES_EXT_TCP

FW_SERVICES_DMZ_TCP=""

## Type:	string

#

# see comments for FW_SERVICES_EXT_UDP

FW_SERVICES_DMZ_UDP=""

## Type:	string

#

# see comments for FW_SERVICES_EXT_IP

FW_SERVICES_DMZ_IP=""

## Type:        string

#

# see comments for FW_SERVICES_EXT_RPC

FW_SERVICES_DMZ_RPC=""

## Type:        string

#

# see comments for FW_CONFIGURATIONS_EXT

FW_CONFIGURATIONS_DMZ=""

## Type:	string

#

# see comments for FW_SERVICES_EXT_TCP

FW_SERVICES_INT_TCP="5801 5901 domain http https imap imaps ipp iscsi-target ldap ldaps microsoft-ds mysql netbios-ssn pbs pbs_mom pbs_resmom pbs_sched pop3 pop3s rsync smtp ssh svrloc xdmcp"

## Type:	string

#

# see comments for FW_SERVICES_EXT_UDP

FW_SERVICES_INT_UDP="bootpc bootps domain ipp ipsec-nat-t isakmp netbios-dgm netbios-ns ntp pbs_resmom svrloc tftp xdmcp"

## Type:	string

#

# see comments for FW_SERVICES_EXT_IP

FW_SERVICES_INT_IP="esp"

## Type:        string

#

# see comments for FW_SERVICES_EXT_RPC

FW_SERVICES_INT_RPC="fypxfrd mountd nfs nfs_acl nlockmgr portmap sgi_fam status ypbind yppasswdd ypserv"

## Type:        string

#

# see comments for FW_CONFIGURATIONS_EXT

FW_CONFIGURATIONS_INT="avahi cups dhcp-server sshd vnc-httpd vnc-server xorg-x11-Xvnc ypbind"

## Type: string

#

# Packets to silently drop without log message

#

# Format: space separated list of net,protocol[,port][,sport]

# Example: "0/0,tcp,445 0/0,udp,4662"

#

# The special value _rpc_ is recognized as protocol and means that dport is

# interpreted as rpc service name. See FW_SERVICES_EXT_RPC for

# details.

#

FW_SERVICES_DROP_EXT=""

## Type: string

## Default: 0/0,tcp,113

#

# Packets to silently reject without log message. Common usage is

# TCP port 113 which if dropped would cause long timeouts when

# sending mail or connecting to IRC servers.

#

# Format: space separated list of net,protocol[,dport][,sport]

# Example: "0/0,tcp,113"

#

# The special value _rpc_ is recognized as protocol and means that dport is

# interpreted as rpc service name. See FW_SERVICES_EXT_RPC for

# details.

#

FW_SERVICES_REJECT_EXT="0/0,tcp,113"

## Type: string

## Default:

#

# Services to allow. This is a more generic form of FW_SERVICES_{IP,UDP,TCP}

# and more specific than FW_TRUSTED_NETS

#

# Format: space separated list of net,protocol[,dport[,sport[,flags]]]

# Example: "0/0,tcp,22"

#

# Supported flags are

#   hitcount=NUMBER     : ipt_recent --hitcount parameter

#   blockseconds=NUMBER : ipt_recent --seconds parameter

#   recentname=NAME     : ipt_recent --name parameter

# Example:

#    Allow max three ssh connects per minute from the same IP address:

#      "0/0,tcp,22,,hitcount=3,blockseconds=60,recentname=ssh"

#

# The special value _rpc_ is recognized as protocol and means that dport is

# interpreted as rpc service name. See FW_SERVICES_EXT_RPC for

# details.

#

FW_SERVICES_ACCEPT_EXT=""

## Type:	string

#

# 10.)

# Which services should be accessible from 'trusted' hosts or nets?

#

# Define trusted hosts or networks (doesn't matter whether they are internal or

# external) and the services (tcp,udp,icmp) they are allowed to use. This can

# be used instead of FW_SERVICES_* for further access restriction. Please note

# that this is no replacement for authentication since IP addresses can be

# spoofed. Also note that trusted hosts/nets are not allowed to ping the

# firewall until you also permit icmp.

#

# Format: space separated list of network[,protocol[,port]]

# in case of icmp, port means the icmp type

#

# Example: "172.20.1.1 172.20.0.0/16 1.1.1.1,icmp 2.2.2.2,tcp,22"

#

FW_TRUSTED_NETS=""

## Type:	string

## Default:

#

# 11.)

# Specify which ports are allowed to access unprivileged ports (>1023)

#

# Format: yes, no or space separated list of ports

#

# You may either allow everyone from anyport access to your highports ("yes"),

# disallow anyone ("no"), anyone who comes from a defined port (portnumber or

# known portname). Note that this is easy to circumvent! The best choice is to

# keep this option unset or set to 'no'

#

# defaults to "no" if not set (good choice)

#

# Note: Use of this variable is deprecated and it will likely be

# removed in the future. If you think it should be kept please

# report your use case at

# http://forge.novell.com/modules/xfmod/project/?susefirewall2

#

FW_ALLOW_INCOMING_HIGHPORTS_TCP=""

## Type:	string

## Default:

#

# See FW_ALLOW_INCOMING_HIGHPORTS_TCP

#

# defaults to "no" if not set (good choice)

#

# Note: Use of this variable is deprecated and it will likely be

# removed in the future. If you think it should be kept please

# report your use case at

# http://forge.novell.com/modules/xfmod/project/?susefirewall2

#

FW_ALLOW_INCOMING_HIGHPORTS_UDP=""

## Type:	string

#

# 13.)

# Which services or networks are allowed to be routed through the

# firewall, no matter which zone they are in?

# Requires: FW_ROUTE

#

# With this option you may allow access to e.g. your mailserver. The

# machines must have valid, non-private, IP addresses which were

# assigned to you by your ISP. This opens a direct link to the

# specified network, so please think twice befor using this option!

#

# Format: space separated list of

#    <source network>,<destination network>[,protocol[,port[,flags]]]

#

#  If the protocol is icmp then port is interpreted as icmp type

#

#  The only flag currently supported is 'ipsec' which means to only

#  match packets that originate from an IPsec tunnel

#

# Examples: - "1.1.1.1,2.2.2.2" allow the host 1.1.1.1 to access any

#             service on the host 2.2.2.2

#           - "3.3.3.3/16,4.4.4.4/24" allow the network 3.3.3.3/16

#             to access any service in the network 4.4.4.4/24

#           - "5.5.5.5,6.6.6.6,igmp" allow routing of IGMP messages

#              from 5.5.5.5 to 6.6.6.6

#           - "0/0,0/0,udp,514" always permit udp port 514 to pass

#             the firewall

#           - "192.168.1.0/24,10.10.0.0/16,,,ipsec \

#              10.10.0.0/16,192.168.1.0/24,,,ipsec" permit traffic

#              from 192.168.1.0/24 to 10.10.0.0/16 and vice versa

#              provided that both networks are connected via an

#              IPsec tunnel.

#           - "fd76:9dbb:91a3:1::/64,fd76:9dbb:91a3:4::/64,tcp,ssh"

#              allow ssh from one IPv6 network to another

#

FW_FORWARD=""

## Type:	string

#

# 13a.)

#

# same as FW_FORWARD but packages are rejected instead of accepted

#

# Requires: FW_ROUTE

#

FW_FORWARD_REJECT=""

## Type:	string

#

# 13b.)

#

# same as FW_FORWARD but packages are dropped instead of accepted

#

# Requires: FW_ROUTE

#

FW_FORWARD_DROP=""

## Type:	string

#

# 14.)

# Which services accessed from the internet should be allowed to masqueraded

# servers (on the internal network or dmz)?

# Requires: FW_ROUTE

#

# With this option you may allow access to e.g. your mailserver. The

# machines must be in a masqueraded segment and may not have public

# IP addesses! Hint: if FW_DEV_MASQ is set to the external interface

# you have to set FW_FORWARD from internal to DMZ for the service as

# well to allow access from internal!

#

# Please note that this should *not* be used for security reasons!

# You are opening a hole to your precious internal network. If e.g.

# the webserver there is compromised - your full internal network is

# compromised!

#

# Format: space separated list of

#    <source network>,<ip to forward to>,<protocol>,<port>[,redirect port,[destination ip]]

#

# Protocol must be either tcp or udp

#

# Examples: - "4.0.0.0/8,10.0.0.10,tcp,80" forward all tcp request on

#             port 80 coming from the 4.0.0.0/8 network to the

#             internal server 10.10.0.10

#           - "4.0.0.0/8,10.0.0.10,tcp,80,81" forward all tcp request on

#             port 80 coming from the 4.0.0.0/8 network to the

#             internal server 10.10.0.10 on port 81

#           - "200.200.200.0/24,10.0.0.10,tcp,80,81,202.202.202.202"

#             the network 200.200.200.0/24 trying to access the

#             address 202.202.202.202 on port 80 will be forwarded

#             to the internal server 10.0.0.10 on port 81

#

# Note: du to inconsitent iptables behaviour only port numbers are possible but

# no service names (https://bugzilla.netfilter.org/bugzilla/show_bug.cgi?id=273)

#

FW_FORWARD_MASQ=""

## Type:	string

#

# 15.)

# Which accesses to services should be redirected to a local port on

# the firewall machine?

#

# This option can be used to force all internal users to surf via

# your squid proxy, or transparently redirect incoming webtraffic to

# a secure webserver.

#

# Format: list of <source network>[,<destination network>,<protocol>[,dport[:lport]]

# Where protocol is either tcp or udp. dport is the original

# destination port and lport the port on the local machine to

# redirect the traffic to

#

# An exclamation mark in front of source or destination network

# means everything EXCEPT the specified network

#

# Example: "10.0.0.0/8,0/0,tcp,80,3128 0/0,172.20.1.1,tcp,80,8080"

#

# Note: contrary to previous SuSEfirewall2 versions it is no longer necessary

# to additionally open the local port

FW_REDIRECT=""

## Type:	yesno

## Default:	yes

#

# 16.)

# Which kind of packets should be logged?

#

# When set to "yes", packages that got dropped and are considered

# 'critical' will be logged. Such packets include for example

# spoofed packets, tcp connection requests and certain icmp types.

#

# defaults to "yes" if not set

#

FW_LOG_DROP_CRIT="yes"

## Type:	yesno

## Default:	no

#

# whether all dropped packets should be logged

#

# Note: for broadcasts to be logged you also need to set

# FW_IGNORE_FW_BROADCAST_* to 'no'

#

# defaults to "no" if not set

#

FW_LOG_DROP_ALL="no"

## Type:	yesno

## Default:	yes

#

# When set to "yes", packages that got accepted and are considered

# 'critical' will be logged. Such packets include for example tcp

# connection requests, rpc connection requests, access to high

# udp/tcp port and forwarded pakets.

#

# defaults to "yes" if not set

#

FW_LOG_ACCEPT_CRIT="yes"

## Type:	yesno

## Default:	no

#

# whether all accepted packets should be logged

#

# Note: setting this to 'yes' causes _LOTS_ of log entries and may

# fill your disk quickly. It also disables FW_LOG_LIMIT

#

# defaults to "no" if not set

#

FW_LOG_ACCEPT_ALL="no"

## Type:	string

#

# How many packets per time unit get logged for each logging rule.

# When empty a default of 3/minute is used to prevent port scans

# flooding your log files. For desktop usage it's a good idea to

# have the limit, if you are using logfile analysis tools however

# you might want to disable it.

#

# Set to 'no' to disable the rate limit. Setting FW_LOG_ACCEPT_ALL

# to 'yes' disables this option as well.

#

# Format: a digit and suffix /second, /minute, /hour or /day

FW_LOG_LIMIT=""

## Type:	string

#

# iptables logging option. Must end with --log-prefix and some prefix

# characters

#

# You may specify an alternative logging target by starting the

# string with "-j ". E.g. "-j ULOG --ulog-prefix SFW2"

#

# only change this if you know what you are doing!

FW_LOG=""

## Type:	yesno

## Default:	yes

#

# 17.)

# Do you want to enable additional kernel TCP/IP security features?

# If set to yes, some obscure kernel options are set.

# (icmp_ignore_bogus_error_responses, icmp_echoreply_rate,

#  icmp_destunreach_rate, icmp_paramprob_rate, icmp_timeexeed_rate,

#  ip_local_port_range, log_martians, rp_filter, routing flush,

#  bootp_relay, proxy_arp, secure_redirects, accept_source_route

#  icmp_echo_ignore_broadcasts, ipfrag_time)

#

# Tip: Set this to "no" until you have verified that you have got a

# configuration which works for you. Then set this to "yes" and keep it

# if everything still works. (It should!) ;-)

#

# Choice: "yes" or "no", if not set defaults to "yes"

#

FW_KERNEL_SECURITY="yes"

## Type:	yesno

## Default:	no

#

# 18.)

# Keep the routing set on, if the firewall rules are unloaded?

# REQUIRES: FW_ROUTE

#

# Choices "yes" or "no", if not set defaults to "no"

#

FW_STOP_KEEP_ROUTING_STATE="no"

## Type:	yesno

## Default:	yes

#

# 19.)

# Allow the firewall to reply to icmp echo requests

#

# defaults to "no" if not set

#

FW_ALLOW_PING_FW="yes"

## Type:	yesno

## Default:	no

#

# 19a.)

# Allow hosts in the dmz to be pinged from hosts in other zones even

# if neither FW_FORWARD nor FW_MASQUERADE is set

#

# Requires: FW_ROUTE

#

# defaults to "no" if not set

#

FW_ALLOW_PING_DMZ="no"

## Type:	yesno

## Default:	no

#

# 19b.)

# Allow hosts in the external zone to be pinged from hosts in other

# zones even if neither FW_FORWARD nor FW_MASQUERADE is set

#

# Requires: FW_ROUTE

#

# defaults to "no" if not set

#

FW_ALLOW_PING_EXT="no"

## Type:	yesno

## Default:	yes

#

# 21.)

# Allow ICMP sourcequench from your ISP?

#

# If set to yes, the firewall will notice when connection is choking, however

# this opens yourself to a denial of service attack. Choose your poison.

#

# Defaults to "yes" if not set

#

FW_ALLOW_FW_SOURCEQUENCH=""

## Type:	string(yes,no)

#

# 22.)

# Allow IP Broadcasts?

#

# Whether the firewall allows broadcasts packets.

# Broadcasts are used for e.g. for Netbios/Samba, RIP, OSPF and Games.

#

# If you want to drop broadcasts however ignore the annoying log entries, set

# FW_IGNORE_FW_BROADCAST_* to yes.

#

# Note that if you allow specifc ports here it just means that broadcast

# packets for that port are not dropped. You still need to set

# FW_SERVICES_*_UDP to actually allow regular unicast packets to

# reach the applications.

#

# Format: either

#           - "yes" or "no"

#           - list of udp destination ports

#

# Examples: - "631 137" allow broadcast packets on port 631 and 137

#              to enter the machine but drop any other broadcasts

#           - "yes" do not install any extra drop rules for

#              broadcast packets. They'll be treated just as unicast

#              packets in this case.

#           - "no" drop all broadcast packets before other filtering

#              rules

#

# defaults to "no" if not set

#

FW_ALLOW_FW_BROADCAST_EXT="xdmcp ipp svrloc ntp netbios-ns netbios-dgm"

## Type:	string

#

# see comments for FW_ALLOW_FW_BROADCAST_EXT

FW_ALLOW_FW_BROADCAST_INT="bootps xdmcp ipp svrloc ntp netbios-ns netbios-dgm"

## Type:	string

#

# see comments for FW_ALLOW_FW_BROADCAST_EXT

FW_ALLOW_FW_BROADCAST_DMZ=""

## Type:	string(yes,no)

#

# Suppress logging of dropped broadcast packets. Useful if you don't allow

# broadcasts on a LAN interface.

#

# This setting only affects packets that are not allowed according

# to FW_ALLOW_FW_BROADCAST_*

#

# Format: either

#           - "yes" or "no"

#           - list of udp destination ports

#

# Examples: - "631 137" silently drop broadcast packets on port 631 and 137

#           - "yes" do not log dropped broadcast packets

#           - "no" log all dropped broadcast packets

#

#

# defaults to "no" if not set

FW_IGNORE_FW_BROADCAST_EXT="yes"

## Type:	string

#

# see comments for FW_IGNORE_FW_BROADCAST_EXT

FW_IGNORE_FW_BROADCAST_INT="no"

## Type:	string

#

# see comments for FW_IGNORE_FW_BROADCAST_EXT

FW_IGNORE_FW_BROADCAST_DMZ="no"

## Type:	list(yes,no,int,ext,dmz,)

## Default:	no

#

# 23.)

# Specifies whether routing between interfaces of the same zone should be allowed

# Requires: FW_ROUTE="yes"

#

# Set this to allow routing between interfaces in the same zone,

# e.g. between all internet interfaces, or all internal network

# interfaces.

#

# Caution: Keep in mind that "yes" affects all zones. ie even if you

# need inter-zone routing only in the internal zone setting this

# parameter to "yes" would allow routing between all external

# interfaces as well. It's better to use

# FW_ALLOW_CLASS_ROUTING="int" in this case.

#

# Choice: "yes", "no", or space separate list of zone names

#

# Defaults to "no" if not set

#

FW_ALLOW_CLASS_ROUTING=""

## Type:	string

#

# 25.)

# Do you want to load customary rules from a file?

#

# This is really an expert option. NO HELP WILL BE GIVEN FOR THIS!

# READ THE EXAMPLE CUSTOMARY FILE AT /etc/sysconfig/scripts/SuSEfirewall2-custom

#

#FW_CUSTOMRULES="/etc/sysconfig/scripts/SuSEfirewall2-custom"

FW_CUSTOMRULES=""

## Type:	yesno

## Default:	no

#

# 26.)

# Do you want to REJECT packets instead of DROPing?

#

# DROPing (which is the default) will make portscans and attacks much

# slower, as no replies to the packets will be sent. REJECTing means, that

# for every illegal packet, a connection reject packet is sent to the

# sender.

#

# Choice: "yes" or "no", if not set defaults to "no"

#

# Defaults to "no" if not set

#

# You may override this value on a per zone basis by using a zone

# specific variable, e.g. FW_REJECT_DMZ="yes"

#

FW_REJECT=""

## Type:	yesno

## Default:	no

#

# see FW_REJECT for description

#

# default config file setting is "yes" assuming that slowing down

# portscans is not strictly required in the internal zone even if

# you protect yourself from the internal zone

#

FW_REJECT_INT="yes"

## Type:	string

#

# 27.)

# Tuning your upstream a little bit via HTB (Hierarchical Token Bucket)

# for more information about HTB see http://www.lartc.org

#

# If your download collapses while you have a parallel upload,

# this parameter might be an option for you. It manages your

# upload stream and reserves bandwidth for special packets like

# TCP ACK packets or interactive SSH.

# It's a list of devices and maximum bandwidth in kbit.

# For example, the german TDSL account, provides 128kbit/s upstream

# and 768kbit/s downstream. We can only tune the upstream.

#

# Example:

# If you want to tune a 128kbit/s upstream DSL device like german TDSL set

# the following values:

# FW_HTB_TUNE_DEV="dsl0,125"

# where dsl0 is your pppoe device and 125 stands for 125kbit/s upstream

#

# you might wonder why 125kbit/s and not 128kbit/s. Well practically you'll

# get a better performance if you keep the value a few percent under your

# real maximum upload bandwidth, to prevent the DSL modem from queuing traffic in

# it's own buffers because queing is done by us now.

# So for a 256kbit upstream

#   FW_HTB_TUNE_DEV="dsl0,250"

# might be a better value than "dsl0,256". There is no perfect value for a

# special kind of modem. The perfect value depends on what kind of traffic you

# have on your line but 5% under your maximum upstream might be a good start.

# Everthing else is special fine tuning.

# If you want to know more about the technical background,

# http://tldp.org/HOWTO/ADSL-Bandwidth-Management-HOWTO/

# is a good start

#

FW_HTB_TUNE_DEV=""

## Type:	list(no,drop,reject)

## Default:	drop

#

# 28.)

# What to do with IPv6 Packets?

#

# On older kernels ip6tables was not stateful so it's not possible to implement

# the same features as for IPv4 on such machines. For these there are three

# choices:

#

# - no: do not set any IPv6 rules at all. Your Host will allow any IPv6

#   traffic unless you setup your own rules.

#

# - drop: drop all IPv6 packets.

#

# - reject: reject all IPv6 packets. This is the default if stateful matching is

#   not available.

#

# Disallowing IPv6 packets may lead to long timeouts when connecting to IPv6

# Adresses. See FW_IPv6_REJECT_OUTGOING to avoid this.

#

# Leave empty to automatically detect whether your kernel supports stateful matching.

#

FW_IPv6=""

## Type:	yesno

## Default:	yes

#

# 28a.)

# Reject outgoing IPv6 Packets?

#

# Set to yes to avoid timeouts because of dropped IPv6 Packets. This Option

# does only make sense with FW_IPv6 != no

#

# Defaults to "yes" if not set

#

FW_IPv6_REJECT_OUTGOING=""

## Type:	list(yes,no,int,ext,dmz,)

## Default:	no

#

# 29.)

# Trust level of IPsec packets.

#

# You do not need to change this if you do not intend to run

# services that should only be available trough an IPsec tunnel.

#

# The value specifies how much IPsec packets are trusted. 'int', 'ext' or 'dmz'

# are the respective zones. 'yes' is the same as 'int. 'no' means that IPsec

# packets belong to the same zone as the interface they arrive on.

#

# Note: you still need to explicitely allow IPsec traffic.

# Example:

#   FW_IPSEC_TRUST="int"

#   FW_SERVICES_EXT_IP="esp"

#   FW_SERVICES_EXT_UDP="isakmp"

#   FW_PROTECT_FROM_INT="no"

#

# Defaults to "no" if not set

#

FW_IPSEC_TRUST="no"

## Type:	string

## Default:

#

# 30.)

# Define additional firewall zones

#

# The built-in zones INT, EXT and DMZ must not be listed here. Names

# of additional zones must only contain lowercase ascii characters.

# To define rules for the additional zone, take the approriate

# variable for a built-in zone and substitute INT/EXT/DMZ with the

# name of the additional zone.

#

# Example:

#   FW_ZONES="wlan"

#   FW_DEV_wlan="wlan0"

#   FW_SERVICES_wlan_TCP="80"

#   FW_ALLOW_FW_BROADCAST_wlan="yes"

#

FW_ZONES=""

## Type:	list(yes,no,auto,)

## Default:

#

# 31.)

# Whether to use iptables-batch

#

# iptables-batch commits all rules in an almost atomic way similar

# to iptables-restore. This avoids excessive iptables calls and race

# conditions.

#

# Choice:

#     - yes: use iptables-batch if available and warn if it isn't

#     - no: don't use iptables-batch

#     - auto: use iptables-batch if available, silently fall back to

#       iptables if it isn't

#

# Defaults to "auto" if not set

#

FW_USE_IPTABLES_BATCH=""

## Type:	string

## Default:

#

# 32.)

# Which additional kernel modules to load at startup

#

# Example:

#   FW_LOAD_MODULES="ip_conntrack_ftp ip_nat_ftp"

#

FW_LOAD_MODULES=""

## Type:	string

## Default:

#

# 33.)

# Bridge interfaces without IP address

#

# Traffic on bridge interfaces like the one used by xen appears to

# enter and leave on the same interface. Add such interfaces here in

# order to install special permitting rules for them.

#

# Format: list of interface names separated by space

#

# Example:

#   FW_FORWARD_ALWAYS_INOUT_DEV="xenbr0"

#

FW_FORWARD_ALWAYS_INOUT_DEV=""

FW_SERVICES_ACCEPT_INT="0/0,tcp,103 0/0,udp,103"

FW_SERVICES_ACCEPT_DMZ=""