REGRAS FIREWALL, ROUTES, NAT, MANGLE ETC

# feb/07/2009 13:45:27 by RouterOS 2.9.50

/ ip pool

add name="hs-pool-2" ranges=192.168.0.2-192.168.0.254

/ ip ipsec proposal

add name="default" auth-algorithms=sha1 enc-algorithms=3des lifetime=30m \

lifebytes=0 pfs-group=modp1024 disabled=no

#error exporting /ip telephony region

#error exporting /ip telephony voice-port linejack

#error exporting /ip telephony voice-port phonejack

#error exporting /ip telephony voice-port voicetronix

#error exporting /ip telephony voice-port voip

#error exporting /ip telephony voice-port isdn

#error exporting /ip telephony voice-port zaptel

/ ip telephony gatekeeper

set

#error

/ ip telephony aaa

set

#error

#error exporting /ip telephony numbers

#error exporting /ip telephony codec

/ ip service

set telnet port=23 address=0.0.0.0/0 disabled=no

set ftp port=21 address=0.0.0.0/0 disabled=no

set www port=8008 address=0.0.0.0/0 disabled=no

set ssh port=22 address=0.0.0.0/0 disabled=no

set www-ssl port=443 address=0.0.0.0/0 certificate=none disabled=yes

/ ip upnp

set enabled=no allow-disable-external-interface=yes show-dummy-rule=yes

/ ip arp

/ ip socks

set enabled=no port=1080 connection-idle-timeout=2m max-connections=200

/ ip dns

set primary-dns=201.10.128.3 secondary-dns=201.10.120.3 \

allow-remote-requests=yes cache-size=4048KiB cache-max-ttl=1w

/ ip dns static

/ ip traffic-flow

set enabled=no interfaces=all cache-entries=4k active-flow-timeout=30m \

inactive-flow-timeout=15s

/ ip address

add address=192.168.0.1/24 network=192.168.0.0 broadcast=192.168.0.255 \

interface=Clientes comment="hotspot network" disabled=no

add address=192.168.4.253/24 network=192.168.4.0 broadcast=192.168.4.255 \

interface=DSL1 comment="escola" disabled=no

add address=192.168.2.253/24 network=192.168.2.0 broadcast=192.168.2.255 \

interface=DSL2 comment="jean" disabled=no

add address=192.168.3.0/27 network=192.168.3.0 broadcast=192.168.3.31 \

interface=Webpages comment="charles" disabled=no

add address=192.168.1.253/24 network=192.168.1.0 broadcast=192.168.1.255 \

interface=DSL4 comment="edinei" disabled=no

add address=192.168.9.253/24 network=192.168.9.0 broadcast=192.168.9.255 \

interface=DSL5 comment="gesmar" disabled=no

add address=192.168.6.1/30 network=192.168.6.0 broadcast=192.168.6.3 \

interface=LINUX comment="Linux" disabled=no

/ ip proxy

set enabled=no port=8080 parent-proxy=0.0.0.0:1 maximal-client-connecions=1000 \

maximal-server-connectons=1000

/ ip proxy access

add dst-port=23-25 action=deny comment="block telnet & spam e-mail relaying" \

disabled=no

/ ip accounting

set enabled=no account-local-traffic=no threshold=256

/ ip accounting web-access

set accessible-via-web=no address=0.0.0.0/0

/ ip neighbor discovery

set DSL1 discover=yes

set Clientes discover=yes

set DSL2 discover=yes

set Webpages discover=yes

set DSL4 discover=yes

set DSL5 discover=yes

set LINUX discover=yes

/ ip route

add dst-address=201.7.176.59/32 gateway=192.168.4.254 scope=255 \

target-scope=10 comment="TesteLinkEscola" disabled=no

add dst-address=201.10.209.185/32 gateway=192.168.1.254 scope=255 \

target-scope=10 comment="TesteLinkEdinei" disabled=no

add dst-address=201.10.209.189/32 gateway=192.168.2.254 scope=255 \

target-scope=10 comment="TesteLinkJean" disabled=no

add dst-address=201.10.249.53/32 gateway=192.168.3.254 scope=255 \

target-scope=10 comment="TesteLinkCharles" disabled=no

add dst-address=201.15.2.5/32 gateway=192.168.9.254 scope=255 target-scope=10 \

comment="TesteLinkGesmar" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.4.254 scope=255 target-scope=10 \

routing-mark=DSL1 comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.2.254 scope=255 target-scope=10 \

routing-mark=DSL2 comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.3.254 scope=255 target-scope=10 \

routing-mark=DSL3 comment="" disabled=yes

add dst-address=0.0.0.0/0 gateway=192.168.1.254 scope=255 target-scope=10 \

routing-mark=DSL4 comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.9.254 scope=255 target-scope=10 \

routing-mark=DSL5 comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.4.254 scope=255 target-scope=10 \

comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.1.254 distance=1 scope=255 \

target-scope=10 comment="" disabled=no

add dst-address=0.0.0.0/0 gateway=192.168.2.254 distance=2 scope=255 \

target-scope=10 comment="" disabled=no

/ ip firewall nat

add chain=srcnat action=masquerade src-address=192.168.6.0/30 \

comment="Masquerade Linux" disabled=no

add chain=dstnat action=redirect to-ports=3128 in-interface=Clientes \

src-address=192.168.0.0/24 dst-port=80 protocol=tcp comment="" disabled=no

add chain=srcnat action=src-nat to-addresses=192.168.4.253 to-ports=0-65535 \

connection-mark=DSL1 comment="NOVAS REGRAS BALANCEAMENTO" disabled=no

add chain=srcnat action=src-nat to-addresses=192.168.2.253 to-ports=0-65535 \

connection-mark=DSL2 comment="" disabled=no

add chain=srcnat action=src-nat to-addresses=192.168.9.253 to-ports=0-65535 \

connection-mark=DSL5 comment="" disabled=no

add chain=srcnat action=src-nat to-addresses=192.168.1.253 to-ports=0-65535 \

connection-mark=DSL4 comment="" disabled=no

add chain=srcnat action=src-nat to-addresses=192.168.9.253 to-ports=0-65535 \

connection-mark=DSL5 comment="" disabled=yes

add chain=dstnat action=accept dst-address=207.46.0.0/16 protocol=tcp \

comment="REGRAS NOVAS MSN" disabled=no

add chain=dstnat action=accept dst-address=64.4.0.0/16 protocol=tcp comment="" \

disabled=no

add chain=dstnat action=accept dst-address=65.54.0.0/16 protocol=tcp \

comment="" disabled=no

add chain=pre-hotspot action=accept in-interface=Clientes \

dst-address=200.201.160.0/24 dst-port=80 protocol=tcp hotspot=auth \

comment="Conectividade Social" disabled=no

add chain=pre-hotspot action=accept in-interface=Clientes \

dst-address=200.201.166.0/24 dst-port=80 protocol=tcp hotspot=auth \

comment="" disabled=no

add chain=pre-hotspot action=accept in-interface=Clientes \

dst-address=200.201.173.0/24 dst-port=80 protocol=tcp hotspot=auth \

comment="" disabled=no

add chain=pre-hotspot action=accept in-interface=Clientes \

dst-address=200.201.174.0/24 dst-port=80 protocol=tcp hotspot=auth \

comment="" disabled=no

add chain=pre-hotspot action=redirect to-ports=64873 dst-address=192.168.0.1 \

dst-port=80 protocol=tcp hotspot=auth comment="Paginas de status do \

hotspot" disabled=no

add chain=pre-hotspot action=redirect to-ports=3128 in-interface=Clientes \

dst-port=80 protocol=tcp hotspot=auth comment="Redirecionamento proxy" \

disabled=no

add chain=pre-hotspot action=accept in-interface=Clientes \

dst-address=201.47.187.0/24 dst-port=80 protocol=tcp hotspot=auth \

comment="CredRural" disabled=no

/ ip firewall mangle

add chain=prerouting action=mark-connection new-connection-mark=DSL1 \

passthrough=yes in-interface=Clientes src-address-list=DSL1 comment="NOVAS \

REGRAS BALANCEAMENTO" disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL1 passthrough=no \

in-interface=Clientes src-address-list=DSL1 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL2 \

passthrough=yes in-interface=Clientes src-address-list=DSL2 comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL2 passthrough=no \

in-interface=Clientes src-address-list=DSL2 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL5 \

passthrough=yes in-interface=Clientes src-address-list=DSL5 comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL5 passthrough=no \

in-interface=Clientes src-address-list=DSL5 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL4 \

passthrough=yes in-interface=Clientes src-address-list=DSL4 comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL4 passthrough=no \

in-interface=Clientes src-address-list=DSL4 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL5 \

passthrough=yes in-interface=Clientes src-address-list=DSL5 comment="" \

disabled=yes

add chain=prerouting action=mark-routing new-routing-mark=DSL5 passthrough=no \

in-interface=Clientes src-address-list=DSL5 comment="" disabled=yes

add chain=prerouting action=mark-connection new-connection-mark=DSL1 \

passthrough=yes connection-state=new in-interface=Clientes nth=3,3,0 \

comment="**************************************************" disabled=no

add chain=prerouting action=add-src-to-address-list in-interface=Clientes \

connection-mark=DSL1 address-list=DSL1 address-list-timeout=1d comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL1 passthrough=no \

in-interface=Clientes connection-mark=DSL1 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL2 \

passthrough=yes connection-state=new in-interface=Clientes nth=3,3,1 \

comment="**************************************************" disabled=no

add chain=prerouting action=add-src-to-address-list in-interface=Clientes \

connection-mark=DSL2 address-list=DSL2 address-list-timeout=1d comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL2 passthrough=no \

in-interface=Clientes connection-mark=DSL2 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL5 \

passthrough=yes connection-state=new in-interface=Clientes nth=3,3,2 \

comment="**************************************************" disabled=no

add chain=prerouting action=add-src-to-address-list in-interface=Clientes \

connection-mark=DSL5 address-list=DSL5 address-list-timeout=1d comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL5 passthrough=no \

in-interface=Clientes connection-mark=DSL5 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL4 \

passthrough=yes connection-state=new in-interface=Clientes nth=3,3,3 \

comment="**************************************************" disabled=no

add chain=prerouting action=add-src-to-address-list in-interface=Clientes \

connection-mark=DSL4 address-list=DSL4 address-list-timeout=1d comment="" \

disabled=no

add chain=prerouting action=mark-routing new-routing-mark=DSL4 passthrough=no \

in-interface=Clientes connection-mark=DSL4 comment="" disabled=no

add chain=prerouting action=mark-connection new-connection-mark=DSL5 \

passthrough=yes connection-state=new in-interface=Clientes nth=4,4,3 \

comment="**************************************************" disabled=yes

add chain=prerouting action=add-src-to-address-list in-interface=Clientes \

connection-mark=DSL5 address-list=DSL5 address-list-timeout=1d comment="" \

disabled=yes

add chain=prerouting action=mark-routing new-routing-mark=DSL5 passthrough=no \

in-interface=Clientes connection-mark=DSL5 comment="" disabled=yes

add chain=output action=mark-connection new-connection-mark=proxyfull \

passthrough=yes src-port=3128 protocol=tcp comment="Proxy \

Full************************************" disabled=no

add chain=output action=mark-packet new-packet-mark=proxyfull passthrough=yes \

connection-mark=proxyfull comment="" disabled=no

add chain=output action=return connection-mark=proxyfull comment="" \

disabled=no

add chain=prerouting action=mark-connection new-connection-mark=p2pcontrol \

passthrough=yes p2p=all-p2p comment="Controle P2P" disabled=no

add chain=prerouting action=mark-packet new-packet-mark=markp2p \

passthrough=yes connection-mark=p2pcontrol comment="" disabled=no

/ ip firewall filter

add chain=input action=accept dst-port=1863 protocol=tcp comment="REGRAS NOVAS \

MSN" disabled=no

add chain=input action=accept src-port=1863 protocol=tcp comment="" \

disabled=no

add chain=input action=accept dst-port=443 protocol=tcp comment="" disabled=no

add chain=forward action=accept dst-port=443 protocol=tcp comment="" \

disabled=no

add chain=forward action=drop out-interface=DSL1 dst-port=3128 protocol=tcp \

comment="Propaga o Cache pela Internet" disabled=no

add chain=forward action=drop out-interface=DSL2 dst-port=3128 protocol=tcp \

comment="" disabled=no

add chain=forward action=drop out-interface=Webpages dst-port=3128 \

protocol=tcp comment="" disabled=no

add chain=forward action=jump jump-target=seguranca comment="Seguracao" \

disabled=yes

add chain=input action=jump jump-target=seguranca comment="" disabled=yes

add chain=forward action=jump jump-target=VIRUS comment="VIRUS" disabled=yes

add chain=input action=jump jump-target=VIRUS comment="" disabled=yes

add chain=seguranca action=drop p2p=warez comment="" disabled=no

add chain=seguranca action=drop connection-state=invalid comment="" \

disabled=no

add chain=seguranca action=drop dst-port=0 protocol=tcp comment="" disabled=no

add chain=seguranca action=drop src-port=0 protocol=tcp comment="" disabled=no

add chain=seguranca action=drop dst-port=0 protocol=udp comment="" disabled=no

add chain=seguranca action=drop src-port=0 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop src-port=445 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=445 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop src-port=445 protocol=udp comment="Drop Blaster \

Worm" disabled=no

add chain=VIRUS action=drop dst-port=445 protocol=udp comment="Drop Blaster \

Worm" disabled=no

add chain=VIRUS action=drop src-port=135-139 protocol=tcp comment="" \

disabled=no

add chain=VIRUS action=drop src-port=135-139 protocol=udp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=135-139 protocol=tcp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=135-139 protocol=udp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=593 protocol=tcp comment="________" \

disabled=no

add chain=VIRUS action=drop dst-port=1024-1030 protocol=tcp comment="________" \

disabled=no

add chain=VIRUS action=drop dst-port=1080 protocol=tcp comment="Drop MyDoom" \

disabled=no

add chain=VIRUS action=drop dst-port=1214 protocol=tcp comment="________" \

disabled=no

add chain=VIRUS action=drop dst-port=1363 protocol=tcp comment="ndm requester" \

disabled=no

add chain=VIRUS action=drop dst-port=1364 protocol=tcp comment="ndm server" \

disabled=no

add chain=VIRUS action=drop dst-port=1368 protocol=tcp comment="screen cast" \

disabled=no

add chain=VIRUS action=drop dst-port=1373 protocol=tcp comment="hromgrafx" \

disabled=no

add chain=VIRUS action=drop dst-port=1377 protocol=tcp comment="cichlid" \

disabled=no

add chain=VIRUS action=drop dst-port=2745 protocol=tcp comment="Bagle VIRUS" \

disabled=no

add chain=VIRUS action=drop dst-port=2283 protocol=tcp comment="Drop Dumaru.Y" \

disabled=no

add chain=VIRUS action=drop dst-port=2535 protocol=tcp comment="Drop Beagle" \

disabled=no

add chain=VIRUS action=drop dst-port=2745 protocol=tcp comment="Drop \

Beagle.C-K" disabled=no

add chain=VIRUS action=drop dst-port=3127 protocol=tcp comment="Drop MyDoom" \

disabled=no

add chain=VIRUS action=drop dst-port=3410 protocol=tcp comment="Drop Backdoor \

OptixPro" disabled=no

add chain=VIRUS action=drop dst-port=4444 protocol=tcp comment="Worm" \

disabled=no

add chain=VIRUS action=drop dst-port=4444 protocol=udp comment="Worm" \

disabled=no

add chain=VIRUS action=drop dst-port=5554 protocol=tcp comment="Drop Sasser" \

disabled=no

add chain=VIRUS action=drop dst-port=8866 protocol=tcp comment="Drop Beagle.B" \

disabled=no

add chain=VIRUS action=drop dst-port=9898 protocol=tcp comment="Drop \

Dabber.A-B" disabled=no

add chain=VIRUS action=drop dst-port=10000 protocol=tcp comment="Drop \

Dumaru.Y" disabled=no

add chain=VIRUS action=drop dst-port=10080 protocol=tcp comment="Drop \

MyDoom.B" disabled=no

add chain=VIRUS action=drop dst-port=12345 protocol=tcp comment="Drop NetBus" \

disabled=no

add chain=VIRUS action=drop dst-port=17300 protocol=tcp comment="Drop Kuang2" \

disabled=no

add chain=VIRUS action=drop dst-port=27374 protocol=tcp comment="Drop \

SubSeven" disabled=no

add chain=VIRUS action=drop dst-port=65506 protocol=tcp comment="Drop PhatBot, \

Agobot, Gaobot" disabled=no

add chain=VIRUS action=drop dst-port=513 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=513 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=525 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=525 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=568-569 protocol=tcp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=568-569 protocol=udp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=1512 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1512 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=396 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=396 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1366 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1366 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1416 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1416 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=201-209 protocol=tcp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=201-209 protocol=udp comment="" \

disabled=no

add chain=VIRUS action=drop dst-port=545 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=545 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1381 protocol=udp comment="" disabled=no

add chain=VIRUS action=drop dst-port=1381 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=3031 protocol=tcp comment="" disabled=no

add chain=VIRUS action=drop dst-port=3031 protocol=udp comment="" disabled=no

add chain=forward action=accept src-address=0.0.0.0/0 \

dst-address=192.168.0.0/24 comment="Regras liberar todas as portas" \

disabled=no

add chain=forward action=accept src-address=192.168.0.0/24 comment="" \

disabled=no

add chain=input action=drop in-interface=DSL1 dst-address=192.168.0.0/24 \

dst-port=3128 protocol=tcp comment="Bloqueio Internet ao proxy" \

disabled=no

add chain=input action=drop in-interface=DSL2 dst-address=192.168.0.0/24 \

dst-port=3128 protocol=tcp comment="" disabled=no

add chain=input action=drop in-interface=Webpages dst-address=192.168.0.0/24 \

dst-port=3128 protocol=tcp comment="" disabled=no

add chain=input action=drop in-interface=DSL4 dst-address=192.168.0.0/24 \

dst-port=3128 protocol=tcp comment="" disabled=no

add chain=input action=drop in-interface=DSL5 dst-address=192.168.0.0/24 \

dst-port=3128 protocol=tcp comment="" disabled=no

/ ip firewall address-list

add list=DSL1 address=192.168.4.253 comment="" disabled=no

add list=DSL2 address=192.168.2.253 comment="" disabled=no

add list=DSL3 address=192.168.3.253 comment="" disabled=yes

add list=DSL4 address=192.168.1.253 comment="" disabled=no

add list=DSL5 address=192.168.9.253 comment="" disabled=no

/ ip firewall service-port

set ftp ports=21,20 disabled=no

set tftp ports=69 disabled=no

set irc ports=6667 disabled=no

set h323 disabled=yes

set quake3 disabled=no

set gre disabled=no

set pptp disabled=no

/ ip firewall connection tracking

set enabled=yes tcp-syn-sent-timeout=5s tcp-syn-received-timeout=5s \

tcp-established-timeout=1d tcp-fin-wait-timeout=10s \

tcp-close-wait-timeout=10s tcp-last-ack-timeout=10s \

tcp-time-wait-timeout=10s tcp-close-timeout=10s udp-timeout=10s \

udp-stream-timeout=3m icmp-timeout=10s generic-timeout=10m \

tcp-syncookie=yes

/ ip dhcp-server config

set store-leases-disk=5m

/ ip hotspot

add name="hotspot1" interface=Clientes address-pool=hs-pool-2 profile=hsprof1 \

idle-timeout=5m keepalive-timeout=none addresses-per-mac=6 disabled=no

/ ip hotspot service-port

set ftp ports=21 disabled=no

/ ip hotspot ip-binding

add mac-address=00:05:9E:87:3A:51 address=192.168.10.223 \

to-address=192.168.10.223 type=bypassed comment="AcessoEscolaCaramuru" \

disabled=no

add mac-address=00:05:9E:86:B7:3D address=192.111.24.1 to-address=192.111.24.1 \

type=bypassed comment="Brasilanda" disabled=no

add mac-address=00:E0:4C:81:86:D1 address=192.111.24.2 to-address=192.111.24.2 \

type=bypassed comment="" disabled=no

add mac-address=00:05:95:87:18:CB address=192.111.24.3 to-address=192.111.24.3 \

type=bypassed comment="" disabled=no

add mac-address=00:05:9E:87:3E:85 address=192.111.30.1 to-address=192.111.30.1 \

type=bypassed comment="G Orlando" disabled=no

add mac-address=00:05:9E:87:37:71 address=192.111.36.1 to-address=192.111.36.1 \

type=bypassed comment="Johanes VanVliet" disabled=no

add mac-address=00:05:9E:86:22:23 address=192.168.111.53 \

to-address=192.168.111.53 type=bypassed comment="Angela Posto" disabled=no

add mac-address=00:05:9E:87:26:57 address=192.111.16.1 to-address=192.111.16.1 \

type=bypassed comment="Euripedes" disabled=no

add mac-address=00:05:9E:87:3E:65 address=192.168.5.222 \

to-address=192.168.5.222 type=bypassed comment="Ap wds orlando" \

disabled=no

add mac-address=00:05:9E:87:3A:51 address=192.168.5.223 \

to-address=192.168.5.223 type=bypassed comment="" disabled=no

add mac-address=00:11:6B:3D:B0:93 address=192.111.11.1 to-address=192.111.11.1 \

type=bypassed comment="Contabilidade helder" disabled=no

/ ip hotspot profile

set default name="default" hotspot-address=0.0.0.0 dns-name="" \

html-directory=hotspot rate-limit="" http-proxy=0.0.0.0:0 \

smtp-server=0.0.0.0 login-by=cookie,http-chap http-cookie-lifetime=3d \

split-user-domain=no use-radius=no

add name="hsprof1" hotspot-address=192.168.0.1 dns-name="primenetmtv.com.br" \

html-directory=hotspot rate-limit="" http-proxy=0.0.0.0:0 \

smtp-server=0.0.0.0 login-by=cookie,http-chap http-cookie-lifetime=4h \

split-user-domain=no use-radius=no

/ ip hotspot user profile

set default name="default" idle-timeout=none keepalive-timeout=2m \

status-autorefresh=1m shared-users=1 transparent-proxy=yes \

open-status-page=always advertise=no

add name="perfilresidencial" address-pool=hs-pool-2 idle-timeout=none \

keepalive-timeout=59m status-autorefresh=10m shared-users=2 \

rate-limit="128k/192k 256k/384k 128k/192k 30/30 8" transparent-proxy=yes \

open-status-page=always advertise=no

add name="perfilcomercial" address-pool=hs-pool-2 idle-timeout=none \

keepalive-timeout=59m status-autorefresh=10m shared-users=6 \

rate-limit="128k/256k 256k/512k 128k/256k 30/30 8" transparent-proxy=yes \

open-status-page=always advertise=no

add name="vencido" address-pool=hs-pool-2 idle-timeout=5m keepalive-timeout=2m \

status-autorefresh=1m shared-users=2 rate-limit="128k/192k" \

transparent-proxy=yes open-status-page=always advertise=yes \

advertise-url=aviso.htm advertise-interval=20m advertise-timeout=15s

add name="bloqueado" address-pool=hs-pool-2 idle-timeout=5m \

keepalive-timeout=2m status-autorefresh=1m shared-users=1 \

rate-limit="16k/16k" transparent-proxy=yes open-status-page=always \

advertise=yes advertise-url=bloqueado.htm advertise-interval=1s \

advertise-timeout=immediately

add name="JCInfor" address-pool=hs-pool-2 idle-timeout=none \

keepalive-timeout=2m status-autorefresh=1m shared-users=15 \

rate-limit="192k/256k 192k/350k 192k/256k 30/30 8" transparent-proxy=yes \

open-status-page=always advertise=no

add name="avisos" address-pool=hs-pool-2 idle-timeout=5m keepalive-timeout=2m \

status-autorefresh=1m shared-users=1 rate-limit="128k/192k 156k/256k \

128k/192k 30/30 8" transparent-proxy=yes open-status-page=always \

advertise=yes advertise-url=error2.htm advertise-interval=20m \

advertise-timeout=immediately

add name="testeturbo" address-pool=hs-pool-2 idle-timeout=none \

keepalive-timeout=59m status-autorefresh=10m shared-users=15 \

rate-limit="256k/512k 256k/512k 256k/512k 30/30 8" transparent-proxy=yes \

open-status-page=always advertise=no

/ ip web-proxy

set enabled=yes src-address=0.0.0.0 port=3128 hostname="proxy" \

transparent-proxy=yes parent-proxy=192.168.6.2:5128 \

cache-administrator="webmaster" max-object-size=30000KiB \

cache-drive=system max-cache-size=none max-ram-cache-size=512000KiB

/ ip web-proxy access

add dst-port=23-25 action=deny comment="block telnet & spam e-mail relaying" \

disabled=no

add src-address=192.168.0.0/24 action=allow comment="" disabled=no

add action=deny comment="" disabled=no

/ ip web-proxy cache

add url=":cgi-bin \\\\\?" action=deny comment="don't cache dynamic http pages" \

disabled=no

add url="https://" action=deny comment="don't cache dynamic https pages" \

disabled=no