Aqui está um passo-a-passo explica como ativar nstreme dual !

MikroTik dispositivos: 


I. MikroTik Dispositivo 1 

1.1. Ativando wireless placas 1 e 2:

[admin@MikroTik] > interface enable wlan1

[admin@MikroTik] > interface enable wlan2

1.2. Atribuir endereço IP à placa de rede:

[admin@MikroTik] > ip address add address=192.168.1.1/24 interface=ether1

1.3. Criando bridge interface:

[admin@MikroTik] > interface bridge add

1.4. Adicionando interface Ethernet para a bridge interface:

[admin@MikroTik] > interface bridge port add interface=ether1 bridge=bridge1

1.5. Configurando cartões wireless 1 e 2 a nstreme modo:

[admin@MikroTik] > interface wireless set wlan1 mode=nstreme-dual-slave

[admin@MikroTik] > interface wireless set wlan2 mode=nstreme-dual-

slave

1.6. Criando nstreme interface dupla e fixação Tx e Rx rádios e frequências:

[admin@MikroTik] > interface wireless nstreme-dual add rx-radio=wlan1 tx-radio=wlan2

rx-band=5ghz tx-band=5ghz rx-frequency=5180 tx-frequency=5300

1.7. Adicionando nstreme interface para a Bridge:

[admin@MikroTik] > interface bridge port add interface=nstreme1 bridge=bridge1

1.8. Verificar o endereço MAC da interface nstreme (neste exemplo: 11:11:11:11:11:11):

[admin@MikroTik] > interface wireless nstremedual print

Flags: X  disabled, R  running

0 X  name="nstreme1" mtu=1500 macaddress=11:11:11:11:11:11 arp=enabled

disablerunningcheck=no tx-radio=wlan2 rx-radio=wlan1 remotemac=00:00:00:00:00:00

tx-band=5ghz tx-frequency=5300 rx-band=5ghz rx-frequency=5180

ratesb=1Mbps,2Mbps,5.5Mbps,11Mbps

ratesa/g=6Mbps,9Mbps,12Mbps,18Mbps,24Mbps,36Mbps,48Mbps,54Mbps

framerpolicy=none framerlimit=2560

II. MikroTik Dispositivo 2 

2.1. Ativando wireless placas 1 e 2:

[admin@MikroTik] > interface enable wlan1

[admin@MikroTik] > interface enable wlan2

2.2. Atribuir endereço IP à placa de rede:

[admin@MikroTik] > ip address add address=192.168.1.2/24 interface=ether1

2.3. Criando bridge interface:

[admin@MikroTik] > interface bridge add

2.4. Adicionando interface Ethernet para a bridge interface:

[admin@MikroTik] > interface bridge port add interface=ether1 bridge=bridge1

2.5. Configurando cartões wireless 1 e 2 a nstreme modo:

[admin@MikroTik] > interface wireless set wlan1 mode=nstreme-dual-slave

[admin@MikroTik] > interface wireless set wlan2 mode=nstreme-dual-

slave

2.6. Criando Nstreme interface dupla e fixação Tx e Rx rádios e frequências e definição do endereço MAC da interface remota nstreme (neste exemplo: 11:11:11:11:11:11 [passo 1,8]):

[admin@MikroTik] > interface wireless nstreme-dual add rx-radio=wlan1 tx-radio=wlan2

rx-band=5ghz tx-band=5ghz rx-frequency=5300 tx-frequency=5180

remotemac=11:11:11:11:11:11 disabled=no

2.7. Adicionando nstreme interface para a Bridge:

[admin@MikroTik] > interface bridge port add interface=nstreme1 bridge=bridge1

2.8. Verificar o endereço MAC da interface nstreme (neste exemplo: 22:22:22:22:22:22):

[admin@MikroTik] > interface wireless nstremedual print

Flags: X  disabled, R  running

0  R name="nstreme1" mtu=1500 macaddress=22:22:22:22:22:22 arp=enabled

disablerunningcheck=no tx-radio=wlan2  rx-radio=wlan1 remotemac=11:11:11:11:11:11

tx-band=5ghz tx-frequency=5180 rx-band=5ghz rx-frequency=5300

ratesb=1Mbps,2Mbps,5.5Mbps,11Mbps

ratesa/g=6Mbps,9Mbps,12Mbps,18Mbps,24Mbps,36Mbps,48Mbps,54Mbps

framerpolicy=none framerlimit=2560

III. MikroTik Dispositivo 1 

1.9. Configurando o endereço MAC da interface remota nstreme (neste exemplo: 22:22:22:22:22:22 [passo 2.8]):

[admin@MikroTik] > interface wireless nstreme-dual set nstreme1

remotemac=22:22:22:22:22:22 disabled=no

Material original

http://wiki.mikrotik.com/wiki/Nstreme_dual_Step-by-Step

Traduzindo por:

Paulo Adriano

Fone: (94)9663-6217

E-mail: paulinhoo0@hotmail.com