/ ip firewall filter

add chain=forward action=drop tcp-flags=syn src-address=192.168.0.0/16 \

protocol=tcp packet-mark=!semlimite connection-limit=25,32 \

comment="Limitando numero conexoes simultaneas" disabled=no

add chain=forward action=accept src-address=192.168.0.0/16 dst-port=!0-1000 \

protocol=tcp comment="" disabled=no

add chain=input action=accept connection-state=established comment="Allow \

Established connections" disabled=no

add chain=input action=accept protocol=udp comment="Allow UDP" disabled=no

add chain=input action=drop dst-port=23 protocol=tcp comment="" disabled=no

add chain=input action=accept protocol=icmp comment="Allow ICMP" disabled=no

add chain=input action=accept src-address=192.168.0.0/16 comment="Allow access \

to router from known network" disabled=no

add chain=forward action=drop connection-state=invalid protocol=tcp \

comment="drop invalid connections" disabled=no

add chain=forward action=accept connection-state=established comment="allow \

already established connections" disabled=no

add chain=forward action=accept connection-state=related comment="allow \

related connections" disabled=no

add chain=forward action=drop src-address=0.0.0.0/8 comment="" disabled=no

add chain=forward action=drop dst-address=0.0.0.0/8 comment="" disabled=no

add chain=forward action=drop src-address=127.0.0.0/8 comment="" disabled=no

add chain=forward action=drop dst-address=127.0.0.0/8 comment="" disabled=no

add chain=forward action=drop src-address=224.0.0.0/3 comment="" disabled=no

add chain=forward action=drop dst-address=224.0.0.0/3 comment="" disabled=no

add chain=forward action=jump jump-target=tcp protocol=tcp comment="" \

disabled=no

add chain=forward action=jump jump-target=udp protocol=udp comment="" \

disabled=no

add chain=forward action=jump jump-target=icmp protocol=icmp comment="" \

disabled=no

add chain=tcp action=drop dst-port=69 protocol=tcp comment="deny TFTP" \

disabled=no

add chain=tcp action=drop dst-port=111 protocol=tcp comment="deny RPC \

portmapper" disabled=no

add chain=tcp action=drop dst-port=135 protocol=tcp comment="deny RPC \

portmapper" disabled=no

add chain=tcp action=drop dst-port=137-139 protocol=tcp comment="deny NBT" \

disabled=no

add chain=tcp action=drop dst-port=445 protocol=tcp comment="deny cifs" \

disabled=no

add chain=tcp action=drop dst-port=2049 protocol=tcp comment="deny NFS" \

disabled=no

add chain=tcp action=drop dst-port=12345-12346 protocol=tcp comment="deny \

NetBus" disabled=no

add chain=tcp action=drop dst-port=20034 protocol=tcp comment="deny NetBus" \

disabled=no

add chain=tcp action=drop dst-port=3133 protocol=tcp comment="deny \

BackOriffice" disabled=no

add chain=tcp action=drop dst-port=67-68 protocol=tcp comment="deny DHCP" \

disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=3:0 comment="allow \

established connections" disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=3:1 comment="allow \

already established connections" disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=4:0 comment="allow \

source quench" disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=8:0 comment="allow \

echo request" disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=11:0 comment="allow \

time exceed" disabled=no

add chain=icmp action=accept protocol=icmp icmp-options=12:0 comment="allow \

parameter bad" disabled=no

add chain=icmp action=drop comment="deny all other types" disabled=no

add chain="forward protocol=tcp dst-port=135-139 action=drop" action=accept \

comment="" disabled=no

add chain="forward protocol=udp dst-port=135-139 action=drop" action=accept \

comment="" disabled=no

add chain="forward protocol=tcp dst-port=445-449 action=drop" action=accept \

comment="" disabled=no

add chain="forward protocol=udp dst-port=445-449 action=drop" action=accept \

comment="" disabled=no

add chain=forward action=jump jump-target=sanity-check comment="Sanity Check" \

disabled=no

add chain=sanity-check action=jump jump-target=drop packet-mark=nat-traversal \

comment="Deny illegal NAT traversal" disabled=no

add chain=sanity-check action=add-src-to-address-list protocol=tcp \

psd=20,3s,3,1 address-list=blocked-addr address-list-timeout=1d \

comment="Block port scans" disabled=no

add chain=sanity-check action=add-src-to-address-list \

tcp-flags=fin,psh,urg,!syn,!rst,!ack protocol=tcp \

address-list=blocked-addr address-list-timeout=1d comment="Block TCP Null \

scan" disabled=no

add chain=sanity-check action=add-src-to-address-list \

tcp-flags=!fin,!syn,!rst,!psh,!ack,!urg protocol=tcp \

address-list=blocked-addr address-list-timeout=1d comment="Block TCP Xmas \

scan" disabled=no

add chain=sanity-check action=jump jump-target=drop protocol=tcp \

src-address-list=blocked-addr comment="" disabled=no

add chain=sanity-check action=jump jump-target=drop tcp-flags=rst protocol=tcp \

comment="Drop TCP RST" disabled=no

add chain=sanity-check action=jump jump-target=drop tcp-flags=fin,syn \

protocol=tcp comment="Drop TCP SYN+FIN" disabled=no

add chain=sanity-check action=jump jump-target=drop connection-state=invalid \

comment="Dropping invalid connections at once" disabled=no

add chain=sanity-check action=accept connection-state=established \

comment="Accepting already established connections" disabled=no

add chain=sanity-check action=accept connection-state=related comment="Also \

accepting related connections" disabled=no

add chain=sanity-check action=jump jump-target=drop \

dst-address-type=broadcast,multicast comment="Drop all traffic that goes \

to multicast or broadcast addresses" disabled=no

add chain=sanity-check action=jump jump-target=drop \

src-address-type=broadcast,multicast comment="Drop all traffic that goes \

from multicast or broadcast addresses" disabled=no

add chain=forward action=jump jump-target=restrict-tcp protocol=tcp comment="" \

disabled=no

add chain=forward action=jump jump-target=restrict-udp protocol=udp comment="" \

disabled=no

add chain=forward action=jump jump-target=restrict-ip comment="" disabled=no

add chain=restrict-tcp action=reject reject-with=icmp-network-unreachable \

connection-mark=auth comment="" disabled=no

add chain=restrict-tcp action=jump jump-target=smtp-first-drop \

connection-mark=smtp comment="anti-spam policy" disabled=no

add chain=smtp-first-drop action=add-src-to-address-list \

src-address-list=first-smtp address-list=approved-smtp \

address-list-timeout=0s comment="" disabled=no

add chain=smtp-first-drop action=return src-address-list=approved-smtp \

comment="" disabled=no

add chain=smtp-first-drop action=add-src-to-address-list \

address-list=first-smtp address-list-timeout=0s comment="" disabled=no

add chain=smtp-first-drop action=reject reject-with=icmp-network-unreachable \

comment="" disabled=no

add chain=restrict-tcp action=jump jump-target=drop connection-mark=other-tcp \

comment="" disabled=no

add chain=restrict-udp action=jump jump-target=drop connection-mark=other-udp \

comment="" disabled=no

add chain=restrict-ip action=jump jump-target=drop connection-mark=other \

comment="" disabled=no

add chain=input action=accept src-address-type=local dst-address-type=local \

comment="Allow local traffic \(between router applications\)" disabled=no

add chain=input action=jump jump-target=sanity-check comment="Sanity Check" \

disabled=no

add chain=input action=jump jump-target=drop dst-address-type=!local \

comment="Dropping packets not destined to the router itself, including all \

broadcast traffic" disabled=no

add chain=input action=accept connection-mark=ping limit=5,5 comment="Allow \

pings, but at a very limited rate \(5 per sec\)" disabled=no

add chain=input action=jump jump-target=drop comment="" disabled=no

add chain=dhcp action=accept src-address=0.0.0.0 dst-address=255.255.255.255 \

comment="" disabled=no

add chain=dhcp action=accept src-address=0.0.0.0 dst-address-type=local \

comment="" disabled=no

add chain=dhcp action=accept dst-address-type=local \

src-address-list=local-addr comment="" disabled=no

add chain=local-services action=accept connection-mark=ssh comment="SSH \

\(22/TCP\)" disabled=no

add chain=local-services action=accept connection-mark=dns comment="DNS" \

disabled=no

add chain=local-services action=accept connection-mark=proxy comment="HTTP \

Proxy \(3128/TCP\)" disabled=no

add chain=local-services action=accept connection-mark=winbox comment="Winbox \

\(8291/TCP\)" disabled=no

add chain=local-services action=drop comment="Drop Other Local Services" \

disabled=no

add chain=public-services action=accept connection-mark=ssh comment="SSH \

\(22/TCP\)" disabled=no

add chain=public-services action=accept connection-mark=pptp comment="PPTP \

\(1723/TCP\)" disabled=no

add chain=public-services action=accept connection-mark=gre comment="GRE for \

PPTP" disabled=no

add chain=public-services action=drop comment="Drop Other Public Services" \

disabled=no

add chain=virus action=drop dst-port=135-139 protocol=tcp comment="Drop \

Blaster Worm" disabled=no

add chain=virus action=drop dst-port=135-139 protocol=udp comment="Drop \

Messenger Worm" disabled=no

add chain=virus action=drop dst-port=445 protocol=tcp comment="Drop Blaster \

Worm" disabled=no

add chain=virus action=drop dst-port=445 protocol=udp comment="Drop Blaster \

Worm" disabled=no

add chain=virus action=drop dst-port=593 protocol=tcp comment="________" \

disabled=no

add chain=virus action=drop dst-port=1024-1030 protocol=tcp comment="________" \

disabled=no

add chain=virus action=drop dst-port=1080 protocol=tcp comment="Drop MyDoom" \

disabled=no

add chain=virus action=drop dst-port=1214 protocol=tcp comment="________" \

disabled=no

add chain=virus action=drop dst-port=1363 protocol=tcp comment="ndm requester" \

disabled=no

add chain=virus action=drop dst-port=1364 protocol=tcp comment="ndm server" \

disabled=no

add chain=virus action=drop dst-port=1368 protocol=tcp comment="screen cast" \

disabled=no

add chain=virus action=drop dst-port=1373 protocol=tcp comment="hromgrafx" \

disabled=no

add chain=virus action=drop dst-port=1377 protocol=tcp comment="cichlid" \

disabled=no

add chain=virus action=drop dst-port=1433-1434 protocol=tcp comment="Worm" \

disabled=no

add chain=virus action=drop dst-port=2745 protocol=tcp comment="Bagle Virus" \

disabled=no

add chain=virus action=drop dst-port=2283 protocol=tcp comment="Drop Dumaru.Y" \

disabled=no

add chain=virus action=drop dst-port=2535 protocol=tcp comment="Drop Beagle" \

disabled=no

add chain=virus action=drop dst-port=2745 protocol=tcp comment="Drop \

Beagle.C-K" disabled=no

add chain=virus action=drop dst-port=3127-3128 protocol=tcp comment="Drop \

MyDoom" disabled=no

add chain=virus action=drop dst-port=3410 protocol=tcp comment="Drop Backdoor \

OptixPro" disabled=no

add chain=virus action=drop dst-port=4444 protocol=tcp comment="Worm" \

disabled=no

add chain=virus action=drop dst-port=4444 protocol=udp comment="Worm" \

disabled=no

add chain=virus action=drop dst-port=5554 protocol=tcp comment="Drop Sasser" \

disabled=no

add chain=virus action=drop dst-port=8866 protocol=tcp comment="Drop Beagle.B" \

disabled=no

add chain=virus action=drop dst-port=9898 protocol=tcp comment="Drop \

Dabber.A-B" disabled=no

add chain=virus action=drop dst-port=10000 protocol=tcp comment="Drop \

Dumaru.Y" disabled=no

add chain=virus action=drop dst-port=10080 protocol=tcp comment="Drop \

MyDoom.B" disabled=no

add chain=virus action=drop dst-port=12345 protocol=tcp comment="Drop NetBus" \

disabled=no

add chain=virus action=drop dst-port=17300 protocol=tcp comment="Drop Kuang2" \

disabled=no

add chain=virus action=drop dst-port=27374 protocol=tcp comment="Drop \

SubSeven" disabled=no

add chain=virus action=drop dst-port=65506 protocol=tcp comment="Drop PhatBot, \

Agobot, Gaobot" disabled=no

add chain=forward action=accept connection-state=established comment="allow \

established connections" disabled=no

add chain=forward action=accept connection-state=related comment="allow \

related connections" disabled=no

add chain=forward action=drop connection-state=invalid comment="drop invalid \

connections" disabled=no

add chain=forward action=accept protocol=icmp comment="allow ping" disabled=no

add chain=forward action=accept protocol=udp comment="allow udp" disabled=no

add chain=input action=add-src-to-address-list protocol=tcp psd=21,3s,3,1 \

address-list="port scanners" address-list-timeout=0s comment="" \

disabled=no

add chain=input action=accept tcp-flags=fin,!syn,!rst,!psh,!ack,!urg \

protocol=tcp comment="" disabled=no

add chain=input action=accept tcp-flags=fin,!syn,!rst,!psh,!ack,!urg \

protocol=tcp comment="" disabled=no

add chain=forward action=drop dst-address=200.210.104.201 comment="" \

disabled=no