/ip firewall filter
add action=accept chain=forward comment="Aceitar todos os tipos de conexoes ou pacotes que venham do proxy" disabled=no src-address=192.168.6.0/30

/ip firewall nat

add chain=srcnat src-addressout=192.168.0.0/24-interface=LINK action masquerade

add chain=srcnat src-addressout=192.168.6.0/30-interface=THUNDER action masquerade
add action=dst-nat chain=dstnat comment="Redirecionamento para o Proxy" disabled=no dst-address=!192.168.6.2 dst-port=80 protocol=tcp \
src-address=192.168.0.0/24 to-addresses=192.168.6.2 to-ports=3128

/ip firewall mangle
add action=mark-connection chain=prerouting comment="Squid - Cache Full" disabled=no dscp=12 new-connection-mark=proxy-hits passthrough=yes
add action=mark-packet chain=prerouting comment="" connection-mark=proxy-hits disabled=no new-packet-mark=proxy-squid passthrough=no
add action=mark-connection chain=prerouting comment="Thunder - Cache Full" disabled=no dst-address=192.168.6.2 dst-port=80 new-connection-mark=\
thunder-connection passthrough=yes protocol=tcp
add action=mark-packet chain=prerouting comment="" connection-mark=thunder-connection disabled=no new-packet-mark=thunder-packs passthrough=no protocol=tcp

Agora vamos às marcações dos Pacotes e taxas de transferência:

/queue tree
add burst-limit=0 burst-threshold=0 burst-time=0s disabled=no limit-at=2000000 max-limit=3000000 name=SquidCache-Down packet-mark=proxy-squid parent=\
global-out priority=8 queue=default
add burst-limit=0 burst-threshold=0 burst-time=0s disabled=no limit-at=2000000 max-limit=3000000 name=SquidCache-Up packet-mark=proxy-squid parent=global-in \
priority=8 queue=default
add burst-limit=0 burst-threshold=0 burst-time=0s disabled=yes limit-at=2000000 max-limit=3000000 name=ThunderCache-Down packet-mark=thunder-packs parent=global-out \
priority=8 queue=default
add burst-limit=0 burst-threshold=0 burst-time=0s disabled=yes limit-at=2000000 max-limit=3000000 name=ThunderCache-Up packet-mark=thunder-packs parent=global-in \
priority=8 queue=default