Updating Firmware on the AI108-4958 products

- Download the new firmware to your PC.  Write down the name of the folder where you stored the firmware file and its filename (ex.  C:\Firmware\AirVerxxxx )

- Start an FTP server on your PC.  A free FTP Server is available at the following link: http://www.airaya.com/support/readme/22446688.htm

- Enter the web configuration utility for the AIRAYA Bridge.

- Click the Advanced tab, and then click the Download Updates tab.  The configuration page below will appear.

- Enter the IP address of your FTP server in the Host IP Address window.

- Enter the User Name for your FTP server in the User Name window.

- Enter the proper password for the above User Name in the Password window.

- Enter the path for the firmware file (ex.  C:\Firmware) in the Image Path window.

- Enter the firmware file name (ex. AirVerxxxx ) in the Image Name window.

- Click on the Update Firmware button at the bottom of the screen.

- After the new firmware has loaded, a window with a Reboot button should appear.  Click it to enable the new firmware.

Note:  The Reboot button may not appear due to pop-up blockers etc.  In this case, cycle power on the unit and the new firmware will load.